SAfe mode appears to be the new name for hibernation.

ThinkPad Configuration for DOS:Version 4.38

Utility for DOS:Version 4.38(1XU602US)

Copyright (C) IBM Corp. 1993,2004. All rights reserved.    US Government Users Restricted Rights - Use, duplication or disclosure restricted by GSA ADP Schedule Contract with IBM Corp.

Command Syntax:    PS2      [Parameter1] [Parameter2] [Parameter3]

Following are available for [Parameter1].

For the complete command syntax, type "PS2 ? [Parameter1]".

1.Power Management related

CDSPeed

Command Syntax:	PS2    CDSPeed High|Normal|Silent|DEFAULT

Current State:	Normal

Description:		This setting is common to all power management modes.

High		The CD-ROM drive runs in a performance-optimized mode, operating at the highest speed as much as    possible.

Normal	Normal state(default).

Silent		Select this mode to reduce the noise that the CD-ROM driver generates.    The performance will be lower because the drive will not run at the highest speed as much as possible.

Cover

Command Syntax :	PS2    Cover      Enable|Disable

Current State:	Enable

Description:		Select 'Disable' to disable the suspend mode when LCD is closed.

CPUPower

Command Syntax :	PS2      CPUPower    Auto|Disable

Current State    :	Auto

Description        :	Enables or disables the power saving feature that stops the microprocessor clock automatically when there are no system activities.    Selecting Disable may result in a shorter battery life.    Normally it is not necessary to change this setting.    Do not change it unless you have an in-depth knowledge of the computers and power management.

DEFAULT

Command Syntax:	PS2    DEFAULT

Description:		Use this parameter to initialize the following settings:

- Suspend timer

- Screen off timer

- Hard disk off timer

- Standby timer

- Processor speed

Note: Some options may not be available, depending on your model.

DISK

Command Syntax:	PS2    DISK      0|3|5|10|20

Current State:	Maximum Performance    mode : 20

Maximum Battery Life mode : 3

Customized mode : 3

Description:		When the internal hard disk drive (HDD) is not in use For the specified number of minutes, the HDD will automatically turn itself off to conserve power.    Value 00 disables this feature.    You can change the timer for Customized mode.    The timer for Maximum Performance or Maximum Battery Life mode has the fixed value.

DOCK

Command Syntax:	PS2    DOCK        Nosuspend|Suspend

Current State:	Nosuspend

Description:		Select 'Suspend' to force the system to enter the suspend mode with the docking station attached. In this case, if the hibernation file is present, the system will enter the hibernation mode.

Note: Depending on the combination of the computer and the docking station, the system may not support suspend/hibernation while docked. Refer to the publications that come with the docking station for details.

HSWITCH

Command Syntax:	PS2    HSWITCH      Enable|Disable

Current State:	Disable

Description:		Select 'Enable' to make the system enter the hibernation mode by the power switch.

HTimer

Command Syntax:	PS2    HTimer      0|1|2|3|5|10|15|20|25|30|45|60

Current State:	Maximum Performance    mode : 0

Maximum Battery Life mode : 0

Customized mode : 0

Description:		When the system is inactive for the specified number of minutes, the system will automatically enter hibernation mode to conserve power. Value 00 disables this feature.

You can change the timer for Customized mode.    The timer for Maximum Performance or Maximum Battery Life mode has the fixed value.

The Fn+F12/F9 key combination will force the system into the hibernation mode.

LBattery

Command Syntax:	PS2    LBattery    Suspend|Hibernation

Current State:	Suspend

Description:		Select 'Hibernation' if you need to have the system enter the hibernation mode when the battery becomes low.    Otherwise, system will enter the suspend mode.

LCd

Command Syntax:	PS2    LCd      0|1|2|3|5|10|15|20|25|30|45|60

Current State:	Maximum Performance    mode : 60

Maximum Battery Life mode : 5

Customized mode : 5

Description:		Set this value to the number of minutes that the display remains powered on while no input is made from the keyboard, pointing stick, mouse or external keypad.    Value 00 disables this feature.

You can change the timer for Customized mode.      The timer for Maximum Performance or Maximum Battery Life mode has the fixed value.

Note: The attached external display will also be blanked out by this timer.

ON

Command Syntax:	PS2    ON          [[yyyy/MM/DD] HH:mm:ss|Clear]

where

yyyy = 1980-2079, MM = 01-12,DD = 01-31, and

HH = 00-23, mm = 00-59, ss = 00-59

Current State:	Clear

Description:		Set the time and date at which the system will resume the operation from the suspend mode.

PCIBUSPower

Command Syntax:	PS2      PCIBUSPower    Auto|Disable

Current State:	Auto

Description:		Enables or disables the power saving feature that stops the PCI bus clock automatically when there are no activities.    Selecting Disable may result in a shorter battery life.    Normally it is not necessary to change this setting.    Do not change it unless you have an in-depth knowledge of the computers and power management.

PMode

Command Syntax:	PS2    PMode    Performance|Battery|Custom    [AC|DC]

Current State:	Performance

Description:		Use this parameter to change the power mode.

Performance:	Maximum Performance mode

Battery:	Maximum Battery Life mode

Custom:	Customized mode

When you specify AC or DC parameter, you can change the power mode for the designated power source, regardless of the current power source.

POwer

Command Syntax:	PS2    POwer      0|1|2|3|5|10|15|20|25|30|45|60

Current State:	Maximum Performance    mode : 60

Maximum Battery Life mode : 5

Customized mode : 5

Description:		When the system remains inactive for the specified number of minutes, the system will automatically enter suspend mode to conserve power. Value 00 disables this feature.

You can change the timer for Customized mode.    The timer for Maximum Performance or Maximum Battery Life mode has the fixed value.

The Fn+F4 key combination will force the system into the suspend mode.

To return from suspend mode, close and open the LCD panel, or press Fn key, or press any key on the external keyboard.

Pressing the power switch has no effect in turning off the system during suspend mode. Two beeps will be sounded.

RI

Command Syntax:	PS2    RI    Enable|Disable

Current State:	Enable

Description:		Select 'Enable' to enable the system to resume the operation on the incoming call.

SAfe

Command Syntax:	PS2    SAfe        Enable|Disable

Current State:	Disable

Description :		Select 'Enable' to enable the RediSafe (safe suspend mode).    When you select this parameter, you also need to create the hibernation file by using PS2 HFILE.    Select 'Disable' to make the system enter the suspend mode quickly.

SPEEDSTEP

Command Syntax:	PS2      SPEEDSTEP Performance|Battery|Automatic|Disable

Current State:	Automatic

Description:		SpeedStep is a mobile technology that allows an enabled CPU to operate in multiple performance states.    Use this parameter to set up the SpeedStep mode of the CPU.

Performance - Maximum Performance mode

This is always high performance mode regardless of the power source (AC or battery).

Battery - Battery Optimized mode

This is always low performance mode regardless of the power source (AC or battery).

Automatic - Automatic mode (recommended)

High performance is selected when AC is used and low performance is selected when battery is used.    This is the recommended mode.

Disable - Disabled mode

SpeedStep mode is disabled.

Reversed - Reversed mode

Low performance is selected when    AC is used and high performance is selected when battery is used.    This mode is displayed when other utility programs modify SpeedStep settings.    This cannot be modified by this utility.

SPeed

Command Syntax:	PS2    SPeed	Fixed 	MAX|Medium|Slow

Auto	Medium|Slow

Current State:	Maximum Performance    mode : Fixed MAX

Maximum Battery Life mode : Auto Slow

Customized mode : Fixed MAX

Description:		Select one of the processor speeds from the list:

1. Fixed MAX          (Highest Performance)

2. Auto    Medium

3. Auto    Slow

4. Fixed Medium

5. Fixed Slow

S2H

Command Syntax:	PS2    S2H              30|60|90|Disable

Current State:	Disable

Description:		The parameter will make the system enter the hibernation mode at the expiry of the specified time the system remains in the suspend mode.

TImer

Command Syntax:	PS2    TImer          Suspend|Hibernation

Current State:	Suspend

Description:		Select 'Suspend' when you use 'PS2 POwer MM' command for suspend.    Select 'Hibernation' when you use 'PS2 POwer MM' command for hibernation.

2. Display device related

SCreen

Command Syntax:	PS2    SCreen        LCD|CRT|BOTH

Current State	:	LCD

Description:		Select the type of display device(s) that you will use.

HVEXPansion

Command Syntax:	PS2    HVEXPansion    ON|OFf

Current State:	ON

Description:		Select 'ON' to display 640 by 480 resolution screen on the SVGA LCD.

3.Alarm related

BEEP

Command Syntax:	PS2      BEEP        ON|OFf          [System|Warn|Alarm|Volume|Lidclose]

Current State:	System: ON    Warn: ON    Alarm: ON    Volume: OFf    Lidclose: ON

Description:		Select this parameter to turn on/off the alarm indication.

System	This beep is to notify the event about the power management:

* AC adapter attached/detached

* Power off

* System State Transition

* Invalid Fn key

* Invalid Function

* Standby/Suspend/Hibernation failure

* Battery low Condition    (LED: Yellow or Orange)

Warn	This beep is to warn you about some critical condition detected in the system like:

* Critical Low Battery Condition

* Suspend to Hibernation transition

Alarm	This beep is to warn you about your incorrect action.

Volume	The system beeps when you change the sound volume by pressing the Fn+PgUp/PgDn or volume keys.

Lidclose	This audible notification allows you to know when it is safe and when it is not safe to carry or move your system. It starts only when you close the lid, and stops when your system enters the suspend state. The sound interval is 1 second.    When it no longer makes a sound, you can safely move your ThinkPad.    If the system fails to enter the suspend state when the lid is closed, the notification sound is heard until you open the lid.

Note: This function does not work when:

- <for APM only>    'lid close suspend' is disabled by 'PS2 Cover Disable' command;

- <for ACPI only>    none is selected for lid close action;

- 'system power management beep' is disabled by 'PS2 BEEP OFf System' command;

- the system is muted or the system volume is 0.

4.ThinkPad Setup related

IRQ

Command Syntax:	PS2 ? IRQ

Description:		Display the current IRQ assignments.

Current State:

System IRQ Assignment

IRQ Level

3	4	5		7	9	10	11	15

Serial-A (3 or 4)		X

Parallel    (5 or 7)					O

IR (3, 4, 5, or 7)	O

Second IDE									O

PCI IRQ								O

Where		O: Currently assigned,

X: Will be assigned after "Enable" is selected,

*: Conflict with another device.

DMA

Command Syntax:	PS2 ? DMA

Description:		Display the current DMA assignments.

Current State:

System DMA Assignment

DMA Channel

0	1		3		5	6	7

IR (0, 1, or 3)							X

Parallel (0, 1, or 3)			X

Where		O: Currently assigned,

X: Will be assigned after "Enable" is selected,

*: Conflict with another device.

KRate

Command Syntax:	PS2      KRate    Normal|Fast

Current State:	Fast

Description:		Select keyboard typematic rate.

FNSticky

Command Syntax:	PS2      FNSticky      Enable|Disable

Current State:	Disable

Description:		Select 'Enable' to enable the Fn key operation without holding the Fn-key.

TPOint

Command Syntax:	PS2      TPOint          Enable|Disable|AUTODisable

Current State:	Enable

Description:		Select 'Enable' to use TrackPoint.    When you set AUTODisable mode, the built-in TrackPoint will be automatically disabled if an external mouse is attached when booting.    Please note however that if you disconnect the external mouse during suspend or undock operation, the built-in TrackPoint will not be re-enabled.

PARallel

Command Syntax:	PS2      PARallel      Enable|Disable

ADDress	1|2|3

MOde	Uni|Bi|EPP|ECP

DMA	0|1|3

Current State:	Enable

When "Enable" is selected, following parameters will be effective:

ADDress:	1

MOde:	Bi

DMA:	Disable

Description:		Use this parameter to set up the parallel port.

As for "ADDress",

1 : LPT1 (3BCh, IRQ7)

2 : LPT2 (378h, IRQ7)

3 : LPT3 (278h, IRQ5)

As for "MOde",

Uni : Uni-directional

Bi    : Bi-directional

EPP : Enhanced Parallel Port (Bi-directional)

ECP : Extended Capabilities Port (Bi-directional)

SERA

Command Syntax:	PS2	SERA	Enable|Disable

ADDress	1|2|3|4

Current State:	Disable

When "Enable" is selected, following parameter will be effective:

ADDress: 1

Description:		Use this parameter to set up the serial-A port.

As for "ADDress",

1 : COM1 (3F8h, IRQ4)

2 : COM2 (2F8h, IRQ3)

3 : COM3 (3E8h, IRQ4)

4 : COM4 (2E8h, IRQ3)

IDE2

Command Syntax:	PS2    IDE2    Enable|Disable

Current State:	Enable

Description:		Select 'Enable' to enable second IDE devices such as a CD-ROM drive or a second hard disk drive in the device bay.

Note: The term 'device bay' is the generic name of the ThinkPad computer bay, such as an UltraBay or an UltraslimBay, which can accept such devices as a CD-ROM drive or a DVD drive.

See the user's manual of your computer for the actual name of your bay.

STARTup

Command Syntax:	PS2    STARTup    Enable|Disable

Current State:	Enable

Description:		Select 'Enable' to display the STARTup screen.

PRESENtation

Command Syntax:	PS2    PRESENtation    Enable|Disable

Current State:	Disable

Description:		This command enables or disables the presentation mode so that screen will not be turned off automatically.    When enabled, screen off timer, standby timer, and suspend timer value are ignored until this mode is disabled.    When disabled, original values of these timers are restored.

PCIIRQ

Command Syntax:	PS2	PCIIRQ	Disable

1st IRQ (2nd IRQ) (3rd IRQ) (4th IRQ)

Valid IRQs    :    3|4|5|7|9|10|11|15

Assigned IRQ(s):	11

Description:		This command is to reserve the IRQ(s) for PCI devices.    When 'Disable' is specified, PCI device may not work properly. To see the current system IRQ assignment, type "PS2 ? IRQ".

IR

Command Syntax:	PS2	IR	Enable|Disable

ADDress	1|2|3|4

DMA	0|1|3

IRQ	3|4|5|7

COMADDress	3F8|2F8|3E8|2E8

Current State:	Disable

When "Enable" is selected, following parameter will be effective:

ADDress: 2

DMA: 3

IRQ: 3

COMADDress: 2F8

Description:		Use this parameter to set up the IR port.

As for "ADDress",

1 : COM1 (3F8h, IRQ4)

2 : COM2 (2F8h, IRQ3)

3 : COM3 (3E8h, IRQ4)

4 : COM4 (2E8h, IRQ3)

5.Others

SUSpend

Command Syntax:	PS2      SUSpend

Description:		Use the parameter to suspend the operation.

OFF

Command Syntax:	PS2      OFF

Description:		Use the parameter to suspend the operation.

HIBernation

Command Syntax:	PS2      HIBernation

Description:		Use the parameter to hibernate the system.

TURN

Command Syntax:	PS2      TURN      OFF

Description:		Use the parameter to turn off the system.

BRightness

Command Syntax:	PS2      BRightness      High|Normal

Current State:	Normal

Description:		Use this parameter to set up the brightness control of LCD.    If you select "High", the LCD brightness will be the same regardless of whether the computer is running on battery power or AC operation. If system is running on the battery power, it is recommended to set "Normal" for the conservation of a battery power.